344C5

Popa Iulia

Laboratory 3 - I/O Monitoring (Linux)

Task 01. Rotational delay - IOPS calculations

- Calculate rotational delay

- Divide 5400 RPM by 60 seconds: 5400/60 = 90 RPS

- Convert 1 of 90 to decimal: 1/90 = 0.0111 seconds per Rotation

- Multiply the seconds per rotation by 1000 milliseconds (11.1 MS per rotation).

- Divide the total in half (RD is considered half a revolution around a disk): 11.1/2 = 5.55 MS

- Add an average of 5.55 MS for seek time: 5.55 MS + 5.55 MS = 11.1 MS

- Add 5.55 MS for latency (internal transfer): 11.1 MS + 5.55 MS = 16.65 MS

- Divide 1000 MS by 13 MS per I/O: 1000/16.65 = 60.06 IOPS

Written like this looked pretty nice!

But...

I think the Internet says that it should be between 50 and 75 IOPS, so 60 should be ok!

Task 02. iostat & iotop

iostat -xdm:

iostat -xdm -p sda:

🤨🤨🤨

iostat -x 1 5:

- Monitoring the behaviour with Iostat

Last two outputs:

- 1355.74 rkB/s and 23.50 r/s

1355.74 / 23.50 = 57.69 kB per read

- 104 wkB/s and 15 w/s

104 / 15 = 6.93 kB data per write

read speed >>> write speed

df -kh /dev/loop*:

B. Monitoring the behaviour with Iotop

sudo iotop:

While the script is running, started iotop and ps -a (to find the process PID and PPID for dummy.sh):

Used the kill command for the process that runs the script and also for the parent process:

Task 03. RAM disk

- df -Th:

Created a 1Gb tmpfs ramdisk with an ext4 file system and mounted it at /mnt/ramdisk. Then used df to check:

- Pipe View & RAM Disk

Used the two commands to generate the files:

- pv /dev/urandom | dd of=mnt/ramdisk/rand bs=2048 count=$((512 * 1024 * 1024 / 2048)):

262144+0 records in56MiB/s] [          <=>                                                                                                                                                                ]

262144+0 records out

536870912 bytes (537 MB, 512 MiB) copied, 3,27098 s, 164 MB/s

512MiB 0:00:03 [ 156MiB/s] [           <=>

]

- pv /dev/urandom | dd of=home/iulia/rand bs=2048 count=$((512 * 1024 * 1024 / 2048)):

262144+0 records in44MiB/s] [          <=>                                                                                                                                                                ]

262144+0 records out

536870912 bytes (537 MB, 512 MiB) copied, 3,57407 s, 150 MB/s

512MiB 0:00:03 [ 143MiB/s] [             <=>                                                                                                                                                             ]

Checked using the df command:

Time and speed are approximately equal. In the first case, the speed is bigger than in the second case.

Task 04. Perf & fuzzing

- Fuzzing with AFL

Ran all the commands in the terminal and got the following result:

- Profile AFL

Used the command perf script -F pid (https://linux.die.net/man/1/perf-script):

Tadam! 🎉🎉🎉

Task 05. Feedback